TOWN OF DEPOSIT

DRIVEWAY DESIGN REGULATIONS

Resolutions Approved by the Town Board

July 12, 2004

Requirements shall apply to all new driveways assessing Town Roads in the Town of Deposit. Driveways proposed onto State or County highways require approval from the State Department of Transportation (DOT) or the County Department of Public Works (DPW), respectfully.

A.   Design Standards

  - Access:

The angle of all driveways with Town Highways shall be as close to ninety (90) degrees as practicable for a distance of twenty (20) feet from the edge of the Town Highway driving surface.

The return radius between the edge of the driveway and the edge of the existing Town Highway driving surface shall be a minimum of fifteen (15) feet on each side of driveway.

There shall be a minimum of one hundred and fifty (150) feet of sight distance from the end of the driveway entering a Town Highway.

Sight easement shall be provided across all driveway corners. No obstructions to vision such as shrubbery, brush, trees, earth, or structure, shall be permitted at the road intersections within the triangle formed by the intersections of road center lines and a line drawn between points along such lines thirty (30) feet distant from their point of intersection.

  - Finished Grade:

All access graders within twenty (20) feet of the Town Highway driving surface shall be level to negative slope with a maximum of three percent (3%) negative slope. It is recommended that a grade of no more than twelve percent (12%) be used for the remaining driveway length on order to accommodate emergency vehicle access.

3.  Pavement or driving width shall be a minimum of twelve (12) feet with

recommended maximum of (40) feet.

- Switchbacks shall have a recommended maximum grade of six percent

(6%), and a radius of no less than fifty (50) feet.

- Construction Specifications

  - It is recommended that the driveway bed shall consist of an eight (8) inch minimum depth of well- graded run of bank gravel or stone. The following materials are NOT acceptable; topsoil, shale, clay, mud, plain dirt, trees, brush, or stumps.

  - The roadside culvert pipe (if required) shall be an eighteen (18) inch diameter pipe or as specified by the Town Highway Superintendent, with a minimum length of thirty (30) feet. Owner shall be responsible for the purchase of the pipe; the Town will install with up two-(2) loads of gravel.

- Maintenance

  - It is strongly recommended that driveways be kept in good driving

Condition. In order to accommodate emergency access the Town recommends that all driveways be kept free of large rocks or stones,

Deep ruts, and potholes, and be kept plowed during winter months.

  - Privately owned ditches must be graded for drainage with adequate

culverts. Water shall not be allowed to run down the driveway therefore causing washouts or problems to Town Highways. Owner(s)

may be held financially responsible for all damage to Town Highways.

- Driveway Access Permits for Existing Lots

  - Application for a driveway access permit MUST be submitted in writing to the Town of Deposit Highway Superintendent. The application will be reviewed and a Driveway Access Permit will be issued if these specifications are met.

  - Driveway Access Permits will be granted at the discretion of the Town         of Deposit Highway Superintendent. Such permit shall remain valid for a period of six (6) months, and may be renewed at the discretion of the Highway Superintendent.

  - No building permit application shall be considered by the Code Enforcement Officer until a Driveway Access Permit is issued.

  - No Certificate of Occupancy shall be issued by the Code Enforcement

Officer until after the driveway is completed, inspected, and approved by the Highway Superintendent.

- Access for Newly Created Lots

  - An application proposing a subdivision shall not receive final approval without first meeting with the Town Highway Superintendent

regarding the accessibility of each lot. The application shall provide the Planning Board with information verifying that the Highway Superintendent has inspected the site and has determined that each new lot can be adequately accessed from a Town road.